UNIVERSITY OF GHANA

(All Rights Reserved)

- COURSE CODE AND TITLE:

CPEN 315 – Computer Organization & Architecture

- COURSE INSTRUCTOR

Dr. Robert Sowah

- LAB INSTRUCTOR

Raphael Kuayi

- GROUP MEMBERS:

- Jeffrey Somuah Sarpong

- Glenn AshiteyFio Ashitey

- PROJECT TITLE:

32-Bit CPU Design Using VHDL (Group 7)

# Abstract

This project presents the design and simulation of key components of a 32-bit processor using VHDL. The system includes an Arithmetic Logic Unit (ALU), Data Memory, Register File, Instruction Memory, ALU Control Unit, and Main Control Unit.

Each module was implemented and verified using ModelSim simulations. The results confirm correct arithmetic operations, memory access, instruction decoding, and control signal generation, demonstrating a functional CPU datapath and control structure.

# Introduction

Modern processors rely on the integration of datapath components and control logic to execute instructions efficiently. This project focuses on designing a simplified 32-bit CPU using VHDL to understand how these components interact.

Objectives

  - Design and implement key CPU components in VHDL

  - Simulate and verify each module using ModelSim

  - Demonstrate correct instruction execution behavior

Relevance

This project provides practical insight into processor design, bridging theoretical concepts such as instruction execution, control logic, and datapath architecture with real hardware modeling.

# Architecture Design

The CPU architecture consists of two main parts:

2.1 Datapath Components

  - ALU – Performs arithmetic and logical operations

  - Register File – Stores operands and results

  - Data Memory – Handles read/write operations

  - Instruction Memory – Stores program instructions

2.2 Control Components

  - Main Control Unit – Generates control signals from opcode

  - ALU Control Unit – Determines ALU operation from ALUOp and function field

2.3 Instruction Set

The following instructions were implemented:

  - R-type: add, sub, and, or, slt

  - I-type: addi, slti, lw, sw, beq

  - J-type: j, jal

# Implementation Details

3.1 Design Methodology

  - Modular design approach

  - Each component implemented independently

  - Verified using dedicated testbenches

  - Integrated conceptually through control signals

3.2 ALU Implementation

  - 32-bit inputs and outputs

  - Operations controlled by 3-bit signal

  - Supports: ADD, SUB, AND, OR, SLT

3.3 Data Memory

  - 256 × 32-bit memory

  - Synchronous read/write

  - Addressed using integer conversion

3.4 Register File

  - 8 registers (32-bit each)

  - 1 write port, 2 read ports

  - Synchronous write, asynchronous read

3.5 Instruction Memory

  - ROM-based design

  - Stores 32-bit MIPS instructions

  - Indexed using program counter (PC)

3.6 ALU Control Unit

  - Inputs: ALUOp and function field

  - Outputs: 3-bit ALU control signal

  - Maps instruction type to ALU operation

3.7 Main Control Unit

- Input: opcode (6 bits)

- Outputs:

  - reg_dst

  - mem_to_reg

  - alu_op

  - jump

  - branch

  - mem_read

  - mem_write

  - alu_src

  - reg_write

  - sign_or_zero

3.8 Simulation Process

  - All modules compiled using ModelSim

  - Signals added to waveform viewer

  - Simulations executed for 100–200 ns

  - Outputs verified against expected values

# Results and Discussion

4.1 ALU Results

  - All arithmetic and logical operations produced correct outputs

  - SLT correctly identified smaller values

4.2 Data Memory Results

  - Values written to memory were correctly retrieved

  - Verified using addresses 2 and 4

4.3 Register File Results

  - Correct storage and retrieval of register values

  - Dual-read functionality verified

4.4 Instruction Memory Results

  - Correct instructions fetched for PC values:

0, 2, 4, 6

4.5 ALU Control Results

  - Correct mapping from ALUOp and funct to ALU control signals

  - All operations matched expected behavior

4.6 Control Unit Results

  - Control signals correctly generated for:

add, slti, j, jal, lw, sw, beq, addi

  - Verified through waveform analysis

4.7 Performance Evaluation

  - All modules operated within expected timing

  - No functional errors observed

  - Minor initialization warnings did not affect results

# Challenges and Solutions

| Challenge | Solution |
| --- | --- |
| Signals showing “No Data” in ModelSim | Added signals before running simulation |
| Metavalue warning (X values) | Initialized signals in testbench |
| Understanding ALU vs ALU Control roles | Separated operation logic from control logic |
| Instruction encoding complexity | Used standard MIPS format |
| Debugging waveform outputs | Used radix conversion (Unsigned/Hex) |

# Conclusion and Future Work

Conclusion

The project successfully implemented a 32-bit CPU datapath and control system using VHDL. All modules functioned correctly and were verified through simulation.

Future Work

  - Integrate all modules into a full CPU

  - Implement pipeline architecture

  - Add hazard detection and forwarding

  - Extend instruction set

  - Implement program execution (multi-cycle)

# References

  - Course Lecture Notes – Computer Organization and Architecture

  - MIPS Architecture Documentation

  - IEEE VHDL Standard Libraries (STD_LOGIC_1164, NUMERIC_STD)

  - ModelSim Simulation Tool Documentation